Computer Networks Lab Report

Submitted By

SELVASUDHARSAN SENGUTTUVAN

RA2411030050014

Under the guidance of

Dr. Mallika

(Assistant Professor, School of Computing)

in partial fulfilment for the award of the degree

of

BACHELOR OF TECHNOLOGY

in

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING WITH CYBER SECURITY

Of

FACULTY OF ENGINEERING AND TECHNOLOGY

SRM INSTITUTE OF SCIENCE AND TECHNOLOGY,

TIRUCHIRAPPALLI

Academic Year: 2026–2027

Date: 23 October 2026

LAB REPORT

Experiment 1: Router Configuration: Passwords & Interface Setup

Tool: Cisco Packet Tracer

# 1. Objective

To configure a Cisco router in Packet Tracer by setting up security passwords (Console, Enable and VTY), configuring IP addresses on the Gigabit Ethernet interfaces, and verifying end-to-end communication across different subnets.

# 2. Network Topology & Addressing Table

## 2.1 Topology Overview

- Router: 1 Cisco ISR4331 (Router0)

- Hosts: 2 PCs (PC0 and PC1)

- Cabling: Copper Cross-Over cables connecting PC0 to Gig0/0/0 and PC1 to Gig0/0/1 (a PC-to-router link uses a cross-over cable).

## 2.2 Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Router0 | Gig0/0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
| Router0 | Gig0/0/1 | 192.168.2.1 | 255.255.255.0 | N/A |
| PC0 | FastEthernet0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| PC1 | FastEthernet0 | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |

# 3. Step-by-Step Implementation

## Step 1: Device Placement & Cabling

- Dragged one router (ISR4331) and two PCs into the Cisco Packet Tracer workspace.

- Connected PC0 to the router's GigabitEthernet0/0/0 interface using a Copper Cross-Over cable.

- Connected PC1 to the router's GigabitEthernet0/0/1 interface using a Copper Cross-Over cable.

## Step 2: Router Password Configuration

Opened the router's CLI tab and configured the management passwords:

Router> enable

Router# configure terminal

! Console Password Setup

Router(config)# line console 0

Router(config-line)# password cisco

Router(config-line)# login

Router(config-line)# exit

! Privileged EXEC Password Setup

Router(config)# enable password cisco

! VTY (Telnet/SSH) Password Setup

Router(config)# line vty 0 4

Router(config-line)# password cisco

Router(config-line)# login

Router(config-line)# exit

## Step 3: Router Interface Configuration

Assigned IP addresses and activated both Gigabit Ethernet interfaces:

! Configure GigabitEthernet 0/0/0 (LAN 1)

Router(config)# interface gigabitEthernet 0/0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

! Configure GigabitEthernet 0/0/1 (LAN 2)

Router(config)# interface gigabitEthernet 0/0/1

Router(config-if)# ip address 192.168.2.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# end

Router# copy running-config startup-config

## Step 4: PC Configuration

Configured static IP settings on both end devices under Desktop > IP Configuration:

- PC0: IP 192.168.1.2, Subnet Mask 255.255.255.0, Default Gateway 192.168.1.1

- PC1: IP 192.168.2.2, Subnet Mask 255.255.255.0, Default Gateway 192.168.2.1

# 4. Screenshot

The figure below shows the Packet Tracer workspace with the topology (PC0 and PC1 connected to Router0) and the router CLI displaying the password and interface configuration, including the link-state messages for GigabitEthernet0/0/0 changing to up.

Figure 1: Network topology and Router0 CLI configuration in Cisco Packet Tracer

# 5. Verification & Testing

- Interface status check: executed show ip interface brief on the router. Both interfaces displayed Status: up and Protocol: up.

- Console password: after exiting EXEC mode (exit), re-entering user mode prompted for the console password (cisco).

- Enable password: entering enable prompted for the privileged mode password (cisco).

- End-to-end connectivity: opened the Command Prompt on PC0 (192.168.1.2) and executed:

PC> ping 192.168.2.2

Result: successful ICMP echo replies were received after the initial ARP resolution (4 packets transmitted, 4 received, 0% loss).

# 6. Conclusion

The network was successfully deployed. The router interfaces were brought up and assigned the correct gateway IP addresses, enabling routing between two distinct subnets (192.168.1.0/24 and 192.168.2.0/24). Router security was established through Console, Enable and VTY passwords

Experiment 2: IP Addressing and Subnetting using Variable Length Subnet Masking (VLSM)

Tool: Cisco Packet Tracer

# 1. Objective

- To build a triangular network topology connecting three Cisco routers in Cisco Packet Tracer.

- To design and calculate appropriate subnets using Variable Length Subnet Masking (VLSM) for point-to-point router links.

- To configure IP addresses on the respective router interfaces and bring them into an operational state.

- To verify interface status and test point-to-point link connectivity using the ping utility.

# 2. Tools & Equipment Required

- Cisco Packet Tracer simulation software

- 3 × Cisco ISR4331 routers (Router0, Router1 and Router2 in Packet Tracer; referred to as R1, R2 and R3 in this report)

- Copper Cross-Over cables for the router-to-router links

# 3. Network Topology & Subnet Design

## 3.1 Topology

The routers are interconnected in a full point-to-point triangle: R1 (Router0) at the top, R2 (Router1) at the bottom left and R3 (Router2) at the bottom right. Link 1 joins R1 and R2, Link 2 joins R1 and R3, and Link 3 joins R2 and R3.

Figure 2: Triangular topology of three routers in Cisco Packet Tracer

## 3.2 VLSM Subnet Calculation

Each point-to-point link requires only 2 usable host IP addresses. A /30 subnet mask (255.255.255.252) provides 2(32−30) − 2 = 2 usable addresses per link, minimizing address waste.

| Link | Network ID | Subnet Mask | Usable Host Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| R1 – R2 | 192.168.1.0/30 | 255.255.255.252 | 192.168.1.1 – 192.168.1.2 | 192.168.1.3 |
| R1 – R3 | 192.168.1.4/30 | 255.255.255.252 | 192.168.1.5 – 192.168.1.6 | 192.168.1.7 |
| R2 – R3 | 192.168.1.8/30 | 255.255.255.252 | 192.168.1.9 – 192.168.1.10 | 192.168.1.11 |

## 3.3 Interface IP Assignment Scheme

| Device | Interface | Connected To | Assigned IP Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| R1 | Gig0/0/0 | R2 (Gig0/0/0) | 192.168.1.1 | 255.255.255.252 |
| R1 | Gig0/0/1 | R3 (Gig0/0/0) | 192.168.1.5 | 255.255.255.252 |
| R2 | Gig0/0/0 | R1 (Gig0/0/0) | 192.168.1.2 | 255.255.255.252 |
| R2 | Gig0/0/1 | R3 (Gig0/0/1) | 192.168.1.9 | 255.255.255.252 |
| R3 | Gig0/0/0 | R1 (Gig0/0/1) | 192.168.1.6 | 255.255.255.252 |
| R3 | Gig0/0/1 | R2 (Gig0/0/1) | 192.168.1.10 | 255.255.255.252 |

# 4. Configuration Steps

## Router 1 (R1) Configuration

Router> enable

Router# configure terminal

Router(config)# hostname R1

R1(config)# interface gigabitEthernet 0/0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.252

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface gigabitEthernet 0/0/1

R1(config-if)# ip address 192.168.1.5 255.255.255.252

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# end

R1# copy running-config startup-config

## Router 2 (R2) Configuration

Router> enable

Router# configure terminal

Router(config)# hostname R2

R2(config)# interface gigabitEthernet 0/0/0

R2(config-if)# ip address 192.168.1.2 255.255.255.252

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# interface gigabitEthernet 0/0/1

R2(config-if)# ip address 192.168.1.9 255.255.255.252

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# end

R2# copy running-config startup-config

## Router 3 (R3) Configuration

Router> enable

Router# configure terminal

Router(config)# hostname R3

R3(config)# interface gigabitEthernet 0/0/0

R3(config-if)# ip address 192.168.1.6 255.255.255.252

R3(config-if)# no shutdown

R3(config-if)# exit

R3(config)# interface gigabitEthernet 0/0/1

R3(config-if)# ip address 192.168.1.10 255.255.255.252

R3(config-if)# no shutdown

R3(config-if)# exit

R3(config)# end

R3# copy running-config startup-config

# 5. Verification & Testing

## 5.1 Interface Status Check

Executed show ip interface brief on each router to confirm proper IP assignment and operational state (Status: up, Protocol: up). Sample output from R1:

| Interface | IP-Address | OK? | Method | Status | Protocol |
| --- | --- | --- | --- | --- | --- |
| Gig0/0/0 | 192.168.1.1 | YES | manual | up | up |
| Gig0/0/1 | 192.168.1.5 | YES | manual | up | up |

## 5.2 Connectivity Testing (Ping)

Direct point-to-point connections were tested using the ping command:

| From | To | Command | Success Rate |
| --- | --- | --- | --- |
| R1 | R2 | ping 192.168.1.2 | 100% (5/5) |
| R1 | R3 | ping 192.168.1.6 | 100% (5/5) |
| R2 | R1 | ping 192.168.1.1 | 100% (5/5) |
| R2 | R3 | ping 192.168.1.10 | 100% (5/5) |
| R3 | R1 | ping 192.168.1.5 | 100% (5/5) |
| R3 | R2 | ping 192.168.1.9 | 100% (5/5) |

# 6. Result & Conclusion

The triangular 3-router topology was successfully constructed in Cisco Packet Tracer. By utilizing a /30 VLSM subnet mask (255.255.255.252), IP address space was efficiently allocated with zero waste for the point-to-point connections. All router interfaces were brought up, and bidirectional reachability across all adjacent links was verified with a 100% ping success rate.